ELEC 2200-002 Digital Logic Circuits 
Fall 2010

Homework 6 Problems

Assigned 10/21/10, due 10/28/10

Problem 1: A four-variable Boolean function is expressed as a sum of four product terms (or cubes):

F(A,B,C,D)=ABD+ABCD+ABC+ACD

- Using a Karnaugh map, identify which terms are prime implicants

- If you find that some cubes (or terms) are not prime implicants, then use consensus and absorption theorems, to transform the right hand side to a sum of prime implicants.

- Using Karnaugh map, express the function as a sum consisting only of prime implicants.

- Problem 2: Using Karnaugh map, find the minimum sum of products (MSOP) form for the four-variable Boolean function:

- F(A,B,C,D) =  m(5, 7, 10, 11, 13) + d(0, 6, 14, 15)

- Problem 3: Using the consensus theorem and absorption theorem, find all prime implicants (PI) of the function,

- F(A,B,C)=AB+BC+AC

- Show that there are no essential prime implicants (EPI). Find the MSOP form. Illustrate all PI’s on Karnaugh map.

- Problem 4: Use the Quine-McCluskey method to minimize the Boolean function,

- F(A,B,C,D) =  m(0, 1, 3, 4, 5, 6, 7, 11, 14)

- Show the final result on a Karnaugh map.