ELEC 5200-001/6200-001 Computer Architecture and Design 
Fall 2007

Homework 4 Problems

Assigned 10/1/07, due 10/8/07

Problem 1: 	Consider MIPS datapaths. The clock rates for single-cycle and multicycle implementations are 250MHz and 1GHz, respectively. The following subroutine is used for estimating the performances. Here, the argument register $a0 contains a very large positive integer (a million or greater), and $a1, $a2 and $a3 contain integers:

add	$t1, $zero, $zero

add	$t0, $zero, $zero

repeat		beq	$a0, $zero, finish

beq	$a0, $a1, skip

beq	$a0, $a2, skip

beq	$a0, $a3, skip

add	$t0, $t0, $a0

skip		addi	$a0, $a0, –1

jump	repeat

finish		add	$v0, $t0, $zero

Determine:		(a) Average cycles per instruction (CPI) for two datapaths.

(b) How much faster is the multicycle execution of the program 					(ratio of single-cycle to multicycle execution times)?

- Problem 2: 	Sketch a schematic showing a multicycle MIPS datapath for the execution of the jump and link (jal) instruction. In your schematic you may include only those units that participate in the execution of this instruction.

Problem 3:	The control for a multicycle MIPS datapath is implemented using a control ROM and two dispatch ROMs. Suppose that all possible 6-bit opcodes are classified into one of five types:

1. load or store

2. R-type

3. Branch type

4. Jump type

5. Illegal

All illegal opcodes require the same action, i.e., exception, for which the control signals needed by the datapath have been determined. Illustrate what modification you will make to the control FSM state diagram. How should the contents of the two dispatch ROMs be modified to facilitate this change?