ELEC 5200-001/6200-001 (Fall 2014)

Homework 3 Problems

Assigned 9/29/14, due 10/6/14

- Problem 1: Execution times of hardware blocks of a single cycle datapath are as follows:

- Multiplexer			  		  1ps

- Control					  3ps

- Register file read or write			15ps

- Sign extension			  		  1ps

- Shift left by 2			  		  1ps

- Adder or ALU					20ps

- Instruction/data memory read or write	25ps

- All other hardware times can be neglected. Find a lower bound on the cycle time for the program counter.

- Problem 2: Consider the hardware times of major units (others being negligible) in a datapath as:

- Memory access (read or write)	25ps

- ALU					20ps

- Register file (read or write)		15ps

- Determine minimum clock cycle time, average CPI, and average instruction execution time for single-cycle and multi-cycle datapaths for the following program statistics:

| - Instruction type | - Percentage in the program | - Clock cycles used by instruction |  |
| --- | --- | --- | --- |
|  |  | - Single-cycle | - Multi-cycle |
| - lw | - 10 | - 1 | - 5 |
| - sw | - 10 | - 1 | - 4 |
| - branch | - 20 | - 1 | - 3 |
| - jump | - 20 | - 1 | - 2 |
| - register | - 40 | - 1 | - 4 |

Problem 3:

- Suppose an operation involving register file, memory or ALU each takes 1 time unit. Neglecting the time of all other hardware, how much time will each MIPS instruction take on a single-cycle datapath? Consider R-type, lw, sw, beq and j instructions.

- What will be the execution times for MIPS instructions on a 5-cycle multi-cycle datapath using a clock period of 1 time unit?

- A program contains the following mix of instructions:

- lw			15%

- sw			10%

- R-type			45%

- Branch			20%

- Jump			10%

- What is the single-cycle CPU time to multicycle CPU time ratio for running this program on the two datapaths?

Problem 4: Consider two types of MIPS datapaths. The clock rates for single-cycle and multicycle implementations are 250MHz and 1GHz, respectively. The following subroutine is used for estimating the performances. The argument register $a0 contains a very large positive integer (assume several million):

repeat		addi	$a0, $a0, –1

bne	$a0, $zero, repeat

add	$v0, $a0, $zero

Determine:

(a) Average cycles per instruction (CPI) for two datapaths.

(b) How much faster is the multicycle execution of the program (ratio of single-cycle to multicycle execution times)?

- Problem 5: Assume that each pipeline register and the program counter have a delay of 10ps. Times taken by other major hardware units in a MIPS processor are:

Memory read or write		75ps

Register file read and write	50ps

ALU				100ps

Add unit			100ps

All other hardware, including the control and multiplexers, has negligible delays. Then,

- What is the maximum clock frequency for a five-stage MIPS pipeline datapath?

- Neglecting any hazards and resulting stalls, what are the pipeline latency and the cycles per instruction (CPI) for a long instruction sequence?

- Compare this datapath with a single cycle MIPS datapath using similar hardware:

| - Datapath | - Clock cycle time | - Clock frequency | - CPI | - Time per instruction | - Million instructions per second (mips) |
| --- | --- | --- | --- | --- | --- |
| - Single-cycle |  |  |  |  |  |
| - Pipeline |  |  |  |  |  |

- Problem 6: Consider a single-cycle datapath as a one-stage pipeline. It consists of combinational and asynchronous circuitry and a single clocked register, the program counter. Its total cycle time consists of an interval q required by the register and an interval h used by the rest of the circuitry:

h

q

Reg.

- Assume that the hardware delay h can be partitioned into n equal stages separated by clocked registers each having a delay q, the first stage register being the program counter. Neglecting the latency and any hazard penalties, compute the execution time of an instruction by this n-stage pipeline.

- Show that the performance limit for the pipeline when we neglect the latency and hazards is determined by the register delay q. Find an upper bound for the clock frequency for this pipeline datapath.

- For an n-stage pipeline, suppose the average hazard penalty is α(n – 1) cycles per instruction, where 0 < α < 1.0 and n ≥ 1. Find the optimum number of pipeline stages.

- For a hardware design it is estimated that h = 4×q and for the ISA, α ≈ 0.2, what is the optimum number of stages for this pipeline and what is the corresponding speed up over a single-cycle datapath?

- For q = 50ps, tabulate clock cycle time, clock rate, average CPI and performance in million instructions per second (mips) for single-cycle datapath and the pipeline datapath with optimum number of stages found in (d).

Problem 7: The following MIPS instruction sequence is executed on a 5-cycle pipeline datapath, which is implemented with hazard detection and forwarding units.

lw	$s1, 0($s0)

add	$t0, $t0, $s1

lw	$s2, 4($s0)

add	$t1, $t1, $s2

How many bubbles, if any, will be required? Can a compiler improve the performance?

Problem 8: A program consists of two nested loops, with a branch instruction at the end of each loop and no other branch instruction anywhere. The outer loop is executed 10 times and the inner loop 20 times. Determine the prediction accuracy percentage for the three prediction strategies: (a) always predict branch not taken, (b) always predict branch taken, and (c) use a 1-bit history buffer initialized to “taken” state for each branch instruction when executed first time.