ELEC 2200-003 Digital Logic Circuits 
Fall 2011

Classwork 4 Problems

Assigned 9/30/11, due 9/30/11

Answer the following questions, selecting just one answer per question.

1. What types of electronic circuits does a transceiver of a cell phone contain?

- Analog

- Digital

- Memory

- Combination of digital, mixed-signal and RF

- None of the above

2. What are the five building blocks of a digital computer?

- Memory, logic, passive and active components

- Datapath, control, memory, input and output

- Display screen, keyboard, mouse, power supply and cooling fan

- Transistors, chips, resistors, capacitors and inductors

- Hardware, software, firmware, programmer and operator

3.  What is the radix for binary numbers?

- 1

- 2

- 4

- 8

- 10

4.  What format does a modern digital computer use to represent real numbers?

- Signed magnitude

- Decimal

- 1’s complement

- IEEE 754 Standard format

- 2’s complement

5.  How can you tell whether a 2’s complement integer is positive or negative?

- By complementing all bits

- By complementing all bits and then adding 1

- Negative only if adding the number to its magnitude produces 0

- Positive if MSB = 0, negative if MSB = 1

- Negative if MSB = 0, positive if MSB = 1

6. What is the negation rule for 2’s complement numbers?

- Invert the MSB

- Invert all bits

- Invert all bits and add 1

- Change MSB to 1

- Invert all bits and subtract 1

7. What indicates overflow when two n-bit 2’s complement integers are added?

- A carry of 1 is generated when MSB’s are added

- Two numbers being added have opposite signs

- Two numbers being added have the same sign

- Two numbers have the same MSB, which differs from the MSB of sum

- None of the above

8. What is the number of bits needed to hold the product of two n-bit 2’s complement integers?

- n

- More than n

- 2n

- n +1

- n2

9. How many iterations does binary division of n-bit integers require?

- 1

- Many, until the result converges

- n/2

- Exactly n

- n2

10. What defines the Hamming distance between two n-bit binary vectors?

- Cartesian distance between the tips of the two vectors

- Manhattan distance

- 0 ≤ Hamming distance ≤ n

- Number of bits in which the two binary vectors match

- Number of bits in which the two binary vectors differ

11. What is the bias used for the exponent of an IEEE 754 floating point number?

- None

- 255

- – 255

- Biased exponent = true exponent – 127

- Biased exponent = true exponent + 127

12. How will you determine the sign of an IEEE 754 floating point number?

- All numbers above certain threshold are negative

- MSB = 0 for negative, MSB = 1 for positive

- MSB = 0 for positive, MSB = 1 for negative

- Always positive

- Cannot tell

13. Find the decimal value of the following 32-bit floating point number represented in IEEE 754 format: 1 10000001 1000 0000 0000 0000 0000 000

- 7.0

- – 7.0

- Negative 0

- Negative infinity

- – 6.0

14. How are two floating point numbers in IEEE 754 format multiplied by a digital circuit?

- Both numbers converted to integers and multiplied

- Convert multiplier to integer and add multiplicand to itself that many times

- Numbers converted to logarithms and added

- Multiply significands, add exponents, normalize, roundoff, add sign

- Could be difficult

15. Suppose a and b belong to a set that defines a Boolean algebra, which also includes operations · and +. If a + b = a, then identify the identity element for + operation.

- a

- b

- 0

- 1

- Cannot tell

16. For Boolean elements a, b and c, which of the following equations expresses commutativity:

(a) 	a + b = b + a

(b) 	a + (b + c) = (a + b) + c

(c) 	a (b + c) = ab + ac

(d) 	a + a = a

(e) 	a + ab = a

17. For Boolean elements a, b and c, which of the following equations expresses idempotency (invariance):

(a) 	a + b = b + a

(b) 	a + (b + c) = (a + b) + c

(c) 	a (b + c) = ab + ac

(d) 	a + a = a

(e) 	a + ab = a

18. For Boolean elements a, b and c, which of the following equations expresses associativity:

(a) 	a + b = b + a

(b) 	a + (b + c) = (a + b) + c

(c) 	a (b + c) = ab + ac

(d) 	a + a = a

(e) 	a + ab = a

19. For Boolean elements a, b and c, which of the following equations is an example of the absorption theorem:

(a) 	a + b = b + a

(b) 	a + (b + c) = (a + b) + c

(c) 	a (b + c) = ab + ac

(d) 	a + a = a

(e) 	a + ab = a

20. For Boolean elements a, b and c, which of the following equations expresses distributivity:

(a) 	a + b = b + a

(b) 	a + (b + c) = (a + b) + c

(c) 	a (b + c) = ab + ac

(d) 	a + a = a

(e) 	a + ab = a