CSV 881: Low-Power Design 
Fall 2013

Homework 1 Problems

Assigned 22/10/13, due 23/10/13

Answer the following questions by circling or underlining exactly one choice.

- On an average how much power does a large Gray code counter consume in comparison to a binary counter?

Twice		Same		Half		¼		1/n for n-bit counter

- Power consumption of a high performance processor chip is:

10-100 W	Less than 1W		500mW		1kW		0.5-1.0W

- The inversion encoding for a reduced-power n-bit bus works as follows:

All 0 bits are inverted		All 1 bits are inverted		Bits are always inverted

All bits are inverted when number of transitions exceeds n/2	Only half the bits are inverted

- A 1 microfarad capacitor is charged to a voltage 1 volt through a 100 kilo ohm series resistor. What is the energy dissipated in the resistor?

1 watt		1 calorie	1 microjoule		half microjoule		1 microwatt

- A digital circuit dissipates 2 nanojoule of energy per cycle. If the clock rate is 1GHz, what is the power consumption of the circuit?

1 joule		1 nanowatt		1 watt		2 watts		0.5 watt

- Identify the largest component of power in a digital CMOS circuit:

Static		Dynamic	Short-circuit		Leakage	Glitch

- How do glitches affect the power consumption of a digital circuit?

Increase static power				Increase dynamic and short-circuit power

Only increase short-circuit power		Have no influence on power consumption

Radiate energy

- Short-circuit power can be reduced to zero if,

Gates are fast		VDD is below the sum of threshold voltage magnitudes of p and n devices

Clock frequency is very low		Circuit is kept cool		Rise/fall times are small

- Path balancing and glitch filtering are methods for,

Making circuit fast		Reducing noise			Improving Reliability

Reducing dynamic power					Saving leakage of static power

- What is the dynamic power consumption of a 1 million gate VLSI chip for which VDD = 1 volt, average gate capacitance = 1pF, average activity factor (counting both rising and falling transitions) = 10%, clock frequency = 2GHz?

100W		20W		10W		2W		1W

- Which is the most effective method for reducing the power consumption of a CMOS logic circuit?

Increase threshold voltage	Reduce threshold voltage	Reduce clock frequency

Use body bias			Reduce VDD

- What determines the upper bound for VDD?

Device breakdown electric field		Power consumption		Power grid

Threshold voltage			No limit

- What determines the lower bound for VDD?

Device breakdown electric field		Power consumption		Power grid

Threshold voltage			No limit

- How much power does the clock signal consume in a typical CMOS circuit?

1W		10W approx.		10%		40-50%		90%

- Clock gating reduces power consumption in:

Combinational logic		Flip-flops		Clock distribution network

Power grid			Clock buffers