# CHEN3600 – Computer-Aided Chemical Engineering	Spring 2012

# Chemical Engineering Department	HW 5

T.D. Placek 	Auburn University

HW 5 – Finding Roots Using fsolve

- (291) Solve the following set of equations:

x12+x1x2=10

x2+3x1x22=57

Also, prepare a plot of x1 versus x2.

- (298)  Consider the following set of chemical reactions:

2A+B=C

A+D=C

With the following equilibrium constants

K1=cCcA2cB

K2=cCcAcD

Determine the steady state concentrations of all components for the following situation:

K1 = 4x10-4, K2 = 3.7x10-2, cA0 = 50, CB0 = 20, cC0 = 5, cD0 = 10

- (126) A bungee jumpers velocity as a function of time can be represented by:

v(t)=gmCDtanh⁡tgCDm

Answer the following question using SI units:  Given a value of CD = 0.25 kg/m, what jumper mass would be associated with a velocity of 36 m/s at 4 s of time?