Supplemental Take Home Quiz, MECH4240, Fall 2016

Due: Thursday 9/22, 12:30, on paper

- Complete the following table on the computer.   You will need to expand the height of rows in order to fit your entries into the cells.

|  | Figure | Geometric Description | Power Transmission Between: | Key Points in Engineering Design | Force Modelling |
| --- | --- | --- | --- | --- | --- |
| Spur Gears |  | Teeth parallel to cylinder axis | Parallel shafts | Most common type, moderate to low speeds | Radial, tangential, no axial force |
| Helical Gears |  | Teeth that follow on helical paths on the cylinder | Parallel and nonparallel nonintersecting shafts | Quieter operation because teeth load gradually and more teeth in contact, higher speeds, higher loads | Radial, axial and tangential |
| Herringbone Gears |  |  |  |  |  |
| Internal Gears |  |  |  |  |  |
| Bevel Gears |  |  |  |  |  |
| Spiral Bevel Gears |  |  |  |  |  |
| Miter Gears |  |  |  |  |  |
| Hypoid Gears |  |  |  |  |  |
| Worm + Worm Gear |  | A worm is a screw meshing with a spur-like gear called the worm gear |  |  |  |

- Three R. R. Moore test specimens are made from steels having ultimate tensile strength of 95, 185 and 240 ksi.   Estimate the 103 cycle fatigue strength for rotating bending and also the bending endurance limit for each steel.

- A 10 mm diameter steel bar having Su = 1200 MPa and Sy = 950 MPa has a fine-ground surface.  Estimate the bending fatigue strength for (1) 106 or more cycles (2) 2 x 105 cycles. (3) Determine a factor of safety for infinite life if the steel bar has an internal moment M = 500 in-lb and compare to the factor of safety as if it were in static loading with the same internal moment.

- List all the different types of rolling-element bearings.  For a particular application the bearing(s) is(are) expected to see large combined loading (both radial and axial) and substantial moment, in a locating bearing situation and loading in both axial directions.  From the SKF table, choose the best bearing for this situation.