ELEC 5200-001/6200-001 (Fall 2011)

Homework 4 Problems

Assigned 10/03/11, due 10/10/11

Problem 1. Develop a VHDL model for a 4-bit counter with the following specifications:

- Asynchronous active low Reset signal which sets output of the counter to 0

- Synchronous active high Enable signal

- The counter adds 1 on every clock pulse when Enable is active and Reset is inactive

Problem 2. Develop a VHDL model for an 8-bit comparator which compares two 8-bit input signals (din1 and din2) with the following specifications:

- The comparator has three 1-bit outputs: C2, C1 and C0

- If din1 is less than din2, C2 would be assigned to 1; both C1 and C0 would be assigned to logic 0

- If din1 is equal to din2, C1 would be assigned to 1; both C2 and C0 would be assigned to 0

- If din1 is larger than din2, C0 would be assigned to 1; both C2 and C1 would be assigned to 0

Problem 3. Develop a VHDL model for a decoder with the following specifications:

- The decoder has a 2-bit input signal (din) as well as a 4-bit output (dout)

- If din has a value of 00, dout would be assigned to 0001

- If din has a value of 01, dout would be assigned to 0010

- If din has a value of 10, dout would be assigned to 0100

- If din has a value of 11, dout would be assigned to 1000

Note: for problem 1 through 3, please verify the logic function of your VHDL model via ModelSim simulations, write a brief summary about your verification process and submit a hard copy of your VHDL models.